Investigation of a tuned mass damper with piezoelectric stack energy harvester attached to an undamped primary structure under harmonic base excitation

Vu Anh Tuan 1, N.N.Linh 2, Nguyen Van Manh 1, Pham Manh Thang3 and N.D.Anh3,4*

1 1Faculty of Mechanical Engineering, Ha Noi University of Civil Engineering, Vietnam

2 Faculty of Mechanical Engineering, Thuyloi University, Vietnam

3 Faculty of Engineering Mechanics and Automation, University of Engineering and Technology, VNU, Vietnam

4 Institute of Mechanics, Vietnam Academy of Science and Technology, Vietnam

ndanh0403@gmail.com

Abstract. This paper develops the fixed point theory for the optimal design of tuned mass dampers with piezoelectric stack energy harvesters under harmonic base excitation. The existence of two fixed points of the amplitude-frequency curve of the primary structure is proved. Based on the requirements for suppressing the vibration of the primary structure and enlarging the harvested power, the optimal tuning, damping, and resistance ratios are determined. Numerical examinations of the system with the obtained optimal parameters are performed, demonstrating a very good compatibility between theory and computation. More specially, it is shown that not only the vibration of the primary structure is effectively reduced but also a large amount of the harvested energy can be captured in the main resonance domain.

Keywords: tuned mass damper, energy harvesting, piezoelectric stack, undamped primary structure, fixed point, harmonic base excitation

- Introduction

Energy harvesting from vibration suppression devices, specially tuned mass dampers with piezoelectric stack energy harvesters (TMD-PSEH), has become a growing need in recent years. Such devices are so-called dual-function ones that are required to be optimized for both mechanical and electrical characteristic parameters due to the inherent electromechanical coupling effect. Nevertheless, it is not always possible to determine the optimal parameters in explicit form. Even in the case of mechanical TMDs, it is difficult to find optimal tuning and damping ratios except in some special cases, usually when the primary structure is undamped [1-3]. The fixed point theory proposed by Den Hartog [1] is one of the few methods that can provide exact results for these cases. For other cases of mechanical TMDs in general, it is necessary to use analytical approximations such as [4-7]. For electromechanical coupling systems such as TMD-PSEH, it is even more difficult to determine simultaneously the optimal mechanical and electrical parameters. Therefore, previous studies on energy-harvesting TMDs such as [8-10] mainly use approaches of approximate and/or numerical analysis to investigate the basic characteristics of these electromechanical coupling systems.

For the above-mentioned reasons, this paper focuses on developing the fixed point theory to determine the optimal tuning, damping, and resistance ratios of TMD-PSEH attached to an undamped primary structure under base excitation.

- Undamped primary structure with TMD-PSEH under base excitation

Consider the system of the undamped primary structure with TMD-PSEH (hereafter called the TMD-PSEH system) subjected to base excitation as shown in Fig. 1a, where  and  are the mass and spring stiffness of the primary structure,  and  are the mass and linear spring stiffness of a TMD,  is the damping coefficient of TMD viscous damper,  is the relative displacement of mass  and base,  is the relative displacement between  and , respectively. A PSEH is connected to the spring  in series mechanically, and to an electric resistor  in parallel electrically. The harmonic base excitation  with amplitude  and frequency  acting on the spring  has the form

Fig. 1. Schematic of TMD-PSEH system: a) lumped-mass model, b) equilibrium of forces

It is known that the block of the TMD spring and PSEH can be treated as an equivalent spring of two springs  and  connected in series. The electromechanical coupling between the mechanical and electrical parameters of the block is presented by the constitutive equations [11]

where

Physically,  are the deformation of the block and the voltage across the external resistor  under the delivered force  which produces the electric charge ,  are the stiffness, effective electromechanical coupling coefficient, and internal capacitance of piezo stack,  are the equivalent ones of the block, respectively. Accordingly, the lumped-parameter model can be used for modeling the TMD-PSEH system. Namely, applying Newton's law for motions of  and  (see Fig. 1b), and combining with the equation obtained by taking the derivative of Eq. (Kirchhoff's law) , yields the governing equations for the TMD-PSEH system

By setting

where  is the electromechanical coupling coefficient,  is the resistance ratio, and  is the transformed voltage. Then the equation system - is rewritten as

Eqs. - perform a linear system of ordinary differential equations (ODE) of three unknown, mechanical displacements  and electrical voltage . The solutions can be determined using analytical methods. Here we deal with dimensionless outputs, including the mechanical magnification factor  of the primary structure, voltage amplitude , and averaging power in a period , that are respectively obtained by the complex amplitude method as follows

where  is the amplitude of displacement response of the primary structure,  is the amplitude of base excitation as denoted in , and

Eq.  shows that  depends on six parameters including three mechanical parameters , one electrical parameter , one electromechanical coupling parameter , and one excitation parameter  also known as perturbation input. When the electromechanical coupling vanishes, i.e. , Eq.  reduces to the formula of the mechanical TMD system [2, 3] where the relative  only depends on . Although closed-form solutions of the system - are analytically determined, it is necessary to optimize the system's performance for engineering purposes.

- Determination of optimal tuning, damping, and resistance ratios of TMD-PSEH

Generally, two main basic requirements for the effective performance of a TMD-PSEH system are technically posted: the first, perhaps also the priority, is to suppress the vibration of the primary structure, the second is to enlarge as much harvested electric energy as possible. From this, the optimization strategy could be drawn as follows

a) The first requirement involves optimizing the stiffness and damping of the TMD-PSEH, represented by the tuning ratio  and damping ratio . This will be carried out first.

b) The second requirement involves optimizing the output electrical power, this will be carried out later.

Consider the amplitude-frequency curve  given by . From the undamped property of the primary structure, there possibly exist fixed points whose ordinates are independent of TMD damping . Hence, looking for fixed points, if they exist, will be performed first. Based on the methodology of Den Hartog in the fixed-point theory [1], we require that the value of  keeps unchanged for arbitrary values of  at the fixed points. For simplicity,  and  are taken, then substituting them to  gives

which leads to the following biquadratic equation for

Solving  obtains two positive solutions. Moreover, using other specific values of , e.g. , also leads to the same result. That means there exist two fixed points of  whose abscissas are those solutions of . Let’s denote  and  these two fixed points and their coordinates. It is seen from (17) that are functions of ,  which can be formaly presented as (not explicitly shown here due to very long expressions)

Substituting   into   yields the ordinates  as follows (also not explicitly shown here due to very long expressions

Following the suggestion by Den Hartog [1], the optimal tuning ratio  is determined from the condition

Substituting  into (20) leads to the following equation of

where  are functions of . The only real positive root of  comes from the equation . Consequently, the optimal tuning ratio is

Substituting  to , one gets the final results of  and

Using , obtains the relative values of  from

Therefore, the TMD-PSEH also has the same two fixed points as the mechanical TMD. More specially, the abscissas and ordinates of P and Q given by , , respectively, are exactly coincident with that of two fixed points of the mechanical TMD as given in [2, 3]. Besides, it is shown from  that

where  is the optimal tuning ratio of the mechanical TMD derived from the fixed-point theory [2, 3].

Next, to find the optimal damping ratio, it is derived from the requirement that  and  are in the vicinity of two peaks of the curve , preferably if they peak in synchronization. This means

The expression of  can be extracted using  and . Indeed, squaring  leads to

Taking the derivative on both sides of  at P and/or Q with noting , yields

where  is the ordinates  and/or . Solving the quadratic equation  for , one gets two solutions

where

Fig. 2. : continuous line, : dash line, : * line, : x line

At point P, replacing  yields two corresponding solutions  and . At point Q, replacing  yields two corresponding solutions  and . Among four solutions, only two positive ones, i.e.  as illustrated in Fig. 2a, are chosen. Since , then the corresponding fixed points cannot peak synchronously with either of these values. Alternatively, the optimal value of  will be taken as the root mean square of  as proposed by Brock [12] (see Fig. 2b)

So the optimal tuning and damping ratios for TMD-PSEH  and  are given by  and  that are functions of . Among these parameters,  and  are constant for a particular TMD and piezoelectric material. Meanwhile, the resistance ratio , as denoted in , is a function of  ( and  are considered constant). Hence, it is necessary to find a specific value of  that is independent of .

Next, consider the electric part of PSEH that is represented by a resistor-capacitor parallel circuit, as shown in Eq. . Neglecting the coupling effect on the circuit, under excitation by a sinusoidal current source with a magnitude  and frequency , the power dissipated by the resistive load  is as follows [13]

Solving the condition  using , it is found that the circuit has a maximum power output at the optimal resistive load of  [13], regarding . So, one gets the optimal resistance ratio

Substituting  into  and , this will give the final results of  and  which are functions of only two constants  and

The explicit expression of  is not presented here due to being very long. It is shown from  that when  one gets the well-known result of the optimal mechanical TMD [2, 3]

It is also seen from Fig. 2 that the optimal resistance ratio  is very near the minimum of  for the variations of . This means the results of the above optimization procedure are well compliance between mechanical and electrical domains.

- Numerical examination

Here, a numerical investigation of the undamped primary structure with the TMD-PSEH is carried out with (see (22)).

Fig. 3a depicts four curves  with five values of : . Clearly, the curve related to  has the lowest peaks in comparison with the curves related to four other values of . The coordinates of P and Q are P(0.908,6.403) and Q(1.062,6.403), which are very close two the left peak (0.91,6.408) and right peak (1.067,6.407), respectively. Fig. 3b depicts the curves  with and five values of . This case corresponds to the adjustment of the external resistor R for the optimal TMD-PSEH in use. Also, the curve related to  has the lowest peaks in comparison with that related to other values of , i.e. ,  (open circuit, ),  (short circuit, ). For , the corresponding curves have two peaks, one of them is always higher than the two peaks of the optimal curve. The coordinates of the left highest peak related to the open circuit is (0.905,7.075), of the right one related to the short circuit is (1.07,6.964).

Fig. 3.  with: a) varies, b) varies

Fig. 4.  with: a) varies, b)  varies

Fig. 4a depicts the average output power  of TMD-PSEH with  and the variations of .  also has two peaks, whereas they are the same heights as , the left peak is always higher than the right peak as , and vice versa as . It is well-known that open-circuit and short-circuit conditions related to  and  gives no output power. Clearly, the curve related to  is the enveloped curve and shows the largest amount of electrical harvest energy in a period.

Fig. 4b depicts  of the optimal TMD-PSEH with the variations of . Clearly, the higher the value of , the larger the value of . The largest values of  mainly occur at the left peaks. The amplitude of  in the resonance domain, say  (blue and pink regions), especially in the main resonance domain, say  (pink region), is much higher than that outside the resonance domain.

- Conclusion

In the paper, this is the first time the fixed point theory by Den Hartog is developed to find the optimal parameters of a TMD-PSEH. Conclusions that can be drawn are:

(a) First, a model and governing equations of the TMD-PSEH system are presented where the TMD spring and PSEH are connected in series. The expressions of dimensionless outputs, such as the mechanical magnification factor  of the primary structure, voltage amplitude, and averaging power  in a period, are also presented.

(b) The existence of two fixed points P and Q of the curve  has been proved analytically, and they coincide with the two fixed points of the optimal mechanical TMD. Thus, the undamped nature of the main structure leads to the existence of two fixed points, whether mechanical TMD or TMD-PSEH.

(c) Based on the two main technical requirements of suppressing the vibration of the primary structure, and enlarging the harvested power, the optimal tuning, damping and resistance ratios, , respectively, are determined. Their closed-form expressions show the dependence of  on two constant parameters, i.e. mass ratio  and electromechanical coupling coefficient , while .

(d) Numerical investigations of the TMD-PSEH system demonstrates the very good agreement between theory and computation. Two peaks of the curve  are very near P and Q and have almost equal ordinates, and  yields the largest amount of harvested energy in a period with the optimal resistance ratio.

(e) It appears that the fixed point theory has a large potential to exploit further energy-harvesting TMD systems.

- Acknowledgments

This paper is supported by Vietnam Academy of Science and Technology under Grant Code NCVCC 03.01/22-23.

- References

- Den Hartog, J.P.: Mechanical Vibrations. New York: McGraw-Hill (1956).

- Warburton, G.B.: Optimal absorber parameters for various combinations of response and excitation parameters. Eart. Eng. and Struct. Dyn. 10, 381-401 (1982).

- Wong, W.O., Cheung Y.L.: Optimal design of a damped dynamic vibration absorber for vibration control of structure excited by ground motion. Eng. Struct. 30, 282-286 (2008).

- Anh, N.D., Nguyen, N.X.: Extension of equivalent linearization method to design of TMD for linear damped systems. Struct. Cont. and Heal. Mon. 19(6), 565–573 2011.

- Anh, N.D, Nguyen, N.X.: Design of TMD for damped linear structures using the dual criterion of equivalent linearization method. Int. J. of Mech. Sci. 77, 164–170 (2013).

- Anh, N.D, Nguyen, N.X., Hoa, L.T.: Design of three-element dynamic vibration absorber for damped linear structures. J. of Sou. and Vib. 332(19), 4482–4495 (2013).

- Anh, N.D, Nguyen, N.X.,  Quan, N,H.: Global-local approach to the design of dynamic vibration absorber for damped structures. J. of Vib. and Cont. 22(14), 3182–3201 (2014).

- Ali, S.F., Adhikari, S.: Energy Harvesting Dynamic Vibration Absorbers. J. Appl. Mech. 80(4), 041004 (2013).

- Tan, T.,  Yan, Z., Zou, Y., Zhang, W.: Optimal dual-functional design for a piezoelectric autoparametric vibration absorber. Mech. Sys. and Sig. Proc. 123, 513-532 (2019).

- Al-Yafeai, D., Darabseh, T., Mourad, A.H.I.: A State-Of-The-Art Review of Car Suspension-Based Piezoelectric Energy Harvesting Systems. Energies. 13(9), 2336 (2020).

- Lai, Y-A., Kim, J-Y., Yang, C-S.W., Chung, L-L.L.: A low-cost and efficient d33-mode piezoelectric tuned mass damper with simultaneously optimized electrical and mechanical tuning. J. of Int. Mat. Sys. and Str. 32(6), 678–696 (2020).

- Brock, J. E.: A Note on the Damped Vibration Absorber. J. Appl. Mech. 13(4), A284 (1946).

- Dicken, J., Mitcheson, P.D., Stoianov, I., Yeatman, E.M.: Power-Extraction Circuits for Piezoelectric Energy Harvesters in Miniature and Low-Power Applications. IEEE Trans. on Pow. Elec. 27(11), 4514–4529 (2012)